HELP for CRCN GUI

Copyright: SNR lab/Michigan Technological University

http://www.mtu.edu

1. Simulation Process Tutorial

1.1 Simulation Process

1.2 Generate a topology and traffic file

2 Simulation Environment Setting

2.1 Physical Layer Setting

2.2 MAC layer Setting

2.3 Routing Setting

2.4 Other Simulation Setting

2.5 Simulation Result

3. Random Topology/Traffic Generators

3.1 Random Topology Generator

3.2 Random Traffic Generator

# 1. Simulation Process Tutorial

## 1.1 Simulation Process

Under main tab as shown in the following figure,

Step 1: Select the simulation setting in the corresponding PHY/Channel/Spectrum Parameters/MAC/Routing setting. For the user defined Routing and MAC protocols, several default names such as Mac3/Routing1/Routing2 are reserved for the user defined protocols in the GUI. Users only need to use these names for their MAC or Routing protocols.

Step 2: Under Other Simulation Setting, define or specify the number of nodes, simulation stop time, and mobility and traffic file. Then specify the ns working directory in Work Dir and file name of the simulation script in Output sim setting file. The traffic and mobility file can be generated under Topology /Traffic Generator tab. The traffic and mobility file will be stored under the Work dir. After click “Confirm” button,

the simulation script will be generated under the Work dir.

Step3:Under Simulation File dir and Name, choose a simulation script from “Open file from…”, or just type in the simulation file with directory as shown in the following figure, then click “StartSim”, which starts the simulation process. When the simulation is ongoing, the “StartSim” will have a darker color.

Step4: After the simulation finish, click the Interference or Throughput button, then you can see the simulation result.

## 1.2 Generate a topology and traffic file

This GUI is incorporated the topology and traffic generator provided by NS2. Please note that the topology and traffic file are written into directory specified by Work Dir.

Step1: Under Random Topology Generator, fill in the parameters for defining the topology and mobility model. The Work Dir under the “Main” tab should be changed to your ns directory.

Click “Confirm Topology Setting” once you finish.

Step2: Under Random Traffic Generator, fill in the parameters required to generate the traffic, and then click “Confirm Traffic Setting”. The topology and traffic file is generated onto the directory specified by Work Dir. They are merged into the same file.

# 2 Simulation Environment Setting

This GUI provides the cognitive wireless simulation selection based on the wireless simulation TCL script for NS-2 [1].

## 2.1 Physical Layer Setting

A. Physical Layer

Under PHY setting, several options are provided to configure the wireless physical layer.

Propagation

There are several selections for Propagation: TwoRayGround/FreeSpace.

Antenna

The default selection is OmniAntenna, which is the only supported antenna type in NS2. Option is provided here for future extension.

The above options are equivalent to the following code in TCL simulation script:

#Define Options set val(prop) Propagation/TwoRayGround ;# radio-propagation model set val(ant) Antenna/OmniAntenna ;# antenna model

B. Channel/Spectrum Parameter Setting

This section defines the number of interfaces per node and number of channels in the network.

No. of Interfaces:

This field indicates the number of interfaces per node. Default is 1 interface for each node. When this field is larger than 1, the field No. of Channels should be set value equal or larger than the value of No. of Interfaces.

No. of Channels:

This field indicates the number of channels to use in the network.

This section defines some spectrum parameter setting.

Channel Decision Maker:

The option routing means the multi-channels are only visible to routing layer. The option routing/DSA means the multi-channels are visible to both routing and MAC layers.

MaxTxPr:

This field defines the maximum transmission power for the Dynamic Spectrum Access algorithm. NS2 C++ library is changed to read this filed. If MaxTxPr is empty, this option will not affect the simulation.

There is no equivalence to the TCL simulation script for MaxTxPr options.

## 2.2 MAC layer Setting

Several MAC protocols are provided as a selection options: 802_11/Maccon/Macng.

Mac3 can be replaced by users’ new MAC protocol.

The detail TCL setting for CR MAC can be referred to this link.

## 2.3 Routing Setting

Routing Protocol

The routing protocol selection includes AODV/DSR/WCETT[1].

Interface queue type

It has two options DropTail/PriQueue,

Interface queue length

This length can be variable. Default is 50.

Routing1 and Routing2 can be replaced by users’ new routing protocol.

The detail TCL setting for CR Routing can be referred to this link.

## 2.4 Other Simulation Setting

No. of mobile nodes

This field indicates the number of mobile nodes in the simulation scenarios. Please note that this number should be consistent with the number defined in topology and traffic file.

Simulation Stop Time

This field indicates the time when simulation stops.

Work Dir

It is the current NS working directory: XXX/ns-allinone-2.31/ns-2.31. All tcl files generated by this GUI will be stored under this directory.

Output sim setting file(.tcl)

This field saves the simulation setting file into a .tcl file. This simulation file will append with the topology and traffic file defined by user when the confirm button is clicked. After the file appending  action, the complete simulation file has the same name. This file will be generated under the directory specified by Work dir.

Mobility & Traffic file

This field specifies the location of topology and traffic file. User can provide topology and location file or generate it under Topology/Traffic Generator. This topology and traffic file will be appended to the final executable file, whose name is given in Output sim setting file(.tcl).

Simulation File dir and name

This field selects the simulation file. The simulation file can be the one provided by the user or generated through the confirm action.

## 2.5 Simulation Result

Interference

When this button is pressed, a figure will pop up showing the interference information on the selected node and channel, which are selected in the fields of Show interference on Node and On Channel。

Throughput

When this button is clicked, the end to end throughput will display.

# 3. Random Topology/Traffic Generators

The Topology/Traffic Generator incorporated in this GUI is described in the NS2 tutorial [2], which is a component released with NS-2.

## 3.1 Random Topology Generator

Version

This selection contains V1 and V2.

Node Num

This field defines the number of nodes that have topology and movement information.

PauseTime

This field indicates the average pause between movements.

MaxSpeed

This field indicates maximum moving speed of a node.

SimTime

This field defines the simulation stop time.

MaxX

It defines the maximum X coordinates among all the nodes.

MaxY

It defines the maximum Y coordinates among all the nodes.

OutFileName

It defines the output directory and topology file name, which will be used by in the main window. It should be in the same directory as the Workdir.

Workdir

It tells this GUI the working directory of NS-2.

## 3.2 Random Traffic Generator

TafficType

It has two traffic types: CBR and TCP.

Seed

The seed value for this random Traffic Generator.

Connection

This field specifies the maximum number of connections.

Rate

This field specifies the rate of packets.

Previous: Exemplary Demonstrations	Error: Reference source not found  Next: References

Return to Main…